### BIOMODULO Contract Specifications

Ventilated crawl space perfectly isolated from the ground below, made of concrete with BIOMODULO plastic modular formwork produced by Geoplast S.p.A.

a) Realization of a 25 cm thick substrate with supply and installation of rolled gravel.

b) Supply and installation of a 10 cm thick layer of lean concrete (insulation sheets can be used as an alternative).

c) Supply and installation of Geoplast BIOMODULO disposable formworks entirely made of recycled and recyclable polypropylene.

d) Installation of BIOMODULO EXTENSIONS and BIOMODULO FERMAGETTO allowing a side closure, thus preventing the concrete from penetrating inside the air chamber and the possible subdivision in areas of the distribution air chamber.

e) Supply and installation of the distribution reinforcing structure (welded mesh) necessary to withstand operating stresses.

f) If necessary installation of 8 x 2 cm linings placed on the distribution nozzles to create surface drainage channels and hole protection (only for 8 cm thick reinforced concrete)

g) Filling jet of the overlaying cover with concrete having Strength Class Rck' = 250 Kg./cm2 and S4 consistency class, layer thickness 6/8 cm installed with or without the use of pumps.

h) Jet vibration.